ELEC 402 – November 14, 2021

Project 4 Report

Martin Chua – 35713411

15nm FinFET

Area; Delay (avg); Area X Delay. 0.25978 um2; 23.636ps; 6.14 ps*um2.

- NAND3

NAND3 Schematic using default widths (2 fingers)

NAND3 Layout. Layout planned using the stick layout method / euler path taught in class, rather than directly translating from schematic to layout. In hindsight, it’s possible to use 3 fingers for a two shared diffusion in the center.

DRC Results. DRC Summary can be found in Appendix A.

LVS Results pass! (Final LVS results in Appendix C)

Worst case tau testbench schematic with 12 fF capacitor. V2 = V3 = VDD = 1 VDC. V1:

As seen in circuit schematic, worst case is when 1 transistor is off (pulled high) and others are on (pulled low) then the single transistor is also turned on (pulled low) and viceversa, aka 001 -> 000 -> 001. This is because all transistors are in series (pulldown) and parallel (pullup).

Rise and fall time is specified (slew rate) in the assignment as 10ps.

Tau with all parasitics from PEX. TpLH = 25.875 ps. TpHL = 51.17 ps.

This clearly does not match (not equal +-5ps) as specified by the assignment. Widths must be changed. Let’s try doubling the width of the NMOS to 4 fingers (default 2):

I also managed to shrink it a little bit more and re-ran with DRC, LVS, and PEX.

Now, TpLH = 23.872 ps and TpHL = 23.4 ps. They match (equal)!

PEX Netlist.

Avg worst case delay is: 23.636ps.

Thus, with area of 0.25978 um2, the area x delay is 6.14 um2 x ps. It is probably possible to shrink the area even further at the cost of repeatedly fixing DRC errors, but since fixing DRC + changing width + refixing DRC already took me tons of hours I’m going to not do that .

- Sizing and Timing of:

  - The logic function (Boolean) can be obtained by observing the pull-down NMOS. We see A and B in parallel, followed by a series C and D. Hence: Y_bar = (A + B)CD and Y = (AB)_bar + C_bar + D_bar

  - Sizing can be obtained by finding the worst case, then going off from there. We are told NMOS W/L is 4 lambda and PMOS W/L is 8 lambda.

    - NMOS: Worst case is through A or B to C to D. There are three transistors, so all their widths are 4 * 3 = 12 lambda

    - PMOS: Worst case is through B to A. There are two transistors, so A and B are 8 * 2 = 16 lambda. Single branch for C and D makes them 8 lambda.

  - Timing to get worst case tpHL and tpLH:

    - tpHL: For high to low, we need the pull-down network to pull from high to low. So, D should be off initially and on afterwards. For the other ones, we want to maximize capacitance and as many to be up to VDD so that discharging takes the longest, so: ABCD = 1010 -> 1011

    - tlLH:  For low to high, we need the pull-up network to pull from low to high. So, C and D should be on, and either B or A should also be on. This means that the PMOS side is not pulling up initially. As delay propagates, we want the most delay to be at the beginning, so A should be off and B should be on, then go off: ABCD = 0111 -> 0011.

    - Simulation:

15nm pdk circuit:

For the TB images below, V2 pulse (left) and ADE L settings (right):

VDD = V1 = V0 = 1V DC

Output cap = 10fF

High to low: ABCD = 1010 -> 1011:

Low to high: ABCD = 0111 -> 0011:

| TpHL | TpLH |
| --- | --- |
| 42.7ps | 34.2ps |

Schematic Netlist is in Appendix B.

- Transmission Gates

  - Expression for output function in terms of A, B, sel, selB. OUT is C’. A’ is controlled by sel, B’ is controlled by selB. C is A’ and sel or B’ and selB, so: OUT = (Asel’) + (BselB’)

  - Equivalent RC circuit model from A to C, given sel is high. A passes through an inverter, then a transmission gate:

    - Resistances approximation: R_inv = R_TG = R_eqn (L/W), R_inv = R_TG = 12.5k (2/4) = 6.25k ohms

    - C1 Capacitance (Ceff -> 1, Cg -> 2 from formula):

      - Cinv = Ceff(4 + 8) = 12Ceff

      - Ctg = Ceff * 2 * (4) + Cg (4) = 8Ceff + 4Cg

      - C1 = Cinv + Ctg = (20Ceff + 4Cg) 0.1= 2.4 fF

    - C2 Capacitances (Ceff -> 1, Cg -> 2 from formula), x multiplier from load inverter (I use x since I already use f for femto for clarity):

      - Cinv = Cg(4 + 8) = 12Cg * x

      - Ctg = Ceff * 2 * (4) + Cg (4) = 8Ceff + 4Cg

      - C2 = Cinv + Ctg = (8Ceff + 12fCg ) 0.1= (2.4x + 1.6) fF

  - A to C delay. Since the resistances are the same, Elmore delay telmore = RC1 + 2RC2, where R is 6.25 k ohms and C1, C2 are above. This can simplify to telmore = 6.25k * 2.4f + 2*6.25k*(2.4x + 1.6)f = 15 + 30x + 10.4 ps =  25.4 + 30x ps

  - RC Delay, Load is 50fF, R becomes R/x, and C becomes (4 + 8) * x * Ceff + 50fF. Step (0.7) is used because it’s RC delay, which is the most ideal case for the output inverter.

So, trc = 0.7 *6.25k/x (50 + 1.2x)f = 4.375 (1.2x + 50) / x ps

  - Optimal output inverter size to minimize A to OUT: elmore + rc delay and minimize:

25.4 + 30x + 4.375 (1.2x + 50) / x

This is at

So optimal multiplier x (aka f) is 2.7.

Appendix A: DRC Summary

…

ACT009 and NW004A as mentioned can be ignored.

Appendix B: Schematic Netlist of Q2

Appendix C: LVS Report Q1

##################################################

##                                              ##

##         C A L I B R E    S Y S T E M         ##

##                                              ##

##             L V S   R E P O R T              ##

##                                              ##

##################################################

REPORT FILE NAME:         NAND3.lvs.report

LAYOUT NAME:              /ubc/ece/home/ugrads/m/mchuahua/Cadence_Proj4/svdb/NAND3.sp ('NAND3')

SOURCE NAME:              /ubc/ece/home/ugrads/m/mchuahua/Cadence_Proj4/NAND3.src.net ('NAND3')

RULE FILE:                /ubc/ece/home/ugrads/m/mchuahua/Cadence_Proj4/_calibrePEX.rul_

CREATION TIME:            Sat Nov 13 04:25:06 2021

CURRENT DIRECTORY:        /ubc/ece/home/ugrads/m/mchuahua/Cadence_Proj4

USER NAME:                mchuahua

CALIBRE VERSION:          v2018.1_36.27    Tue Apr 3 12:54:13 PDT 2018

OVERALL COMPARISON RESULTS

#       ###################       _   _

#        #                 #       *   *

#   #         #     CORRECT     #         |

# #          #                 #       \___/

#           ###################

**************************************************************************************************************

CELL  SUMMARY

**************************************************************************************************************

Result         Layout                        Source

-----------    -----------                   --------------

CORRECT        NAND3                         NAND3

**************************************************************************************************************

LVS PARAMETERS

**************************************************************************************************************

o LVS Setup:

LVS COMPONENT TYPE PROPERTY            element

LVS COMPONENT SUBTYPE PROPERTY         model

// LVS PIN NAME PROPERTY

LVS POWER NAME                         "VDD"

LVS GROUND NAME                        "VSS"

LVS CELL SUPPLY                        NO

LVS RECOGNIZE GATES                    ALL

LVS IGNORE PORTS                       NO

LVS CHECK PORT NAMES                   NO

LVS IGNORE TRIVIAL NAMED PORTS         NO

LVS BUILTIN DEVICE PIN SWAP            YES

LVS ALL CAPACITOR PINS SWAPPABLE       NO

LVS DISCARD PINS BY DEVICE             NO

LVS SOFT SUBSTRATE PINS                NO

LVS INJECT LOGIC                       NO

LVS EXPAND UNBALANCED CELLS            YES

LVS FLATTEN INSIDE CELL                NO

LVS EXPAND SEED PROMOTIONS             NO

LVS PRESERVE PARAMETERIZED CELLS       NO

LVS GLOBALS ARE PORTS                  YES

LVS REVERSE WL                         NO

LVS SPICE PREFER PINS                  NO

LVS SPICE SLASH IS SPACE               YES

LVS SPICE ALLOW FLOATING PINS          YES

// LVS SPICE ALLOW INLINE PARAMETERS

LVS SPICE ALLOW UNQUOTED STRINGS       NO

LVS SPICE CONDITIONAL LDD              NO

LVS SPICE CULL PRIMITIVE SUBCIRCUITS   NO

// LVS SPICE EXCLUDE CELL SOURCE

// LVS SPICE EXCLUDE CELL LAYOUT

LVS SPICE IMPLIED MOS AREA             NO

// LVS SPICE MULTIPLIER NAME

LVS SPICE OVERRIDE GLOBALS             NO

LVS SPICE REDEFINE PARAM               NO

LVS SPICE REPLICATE DEVICES            NO

LVS SPICE SCALE X PARAMETERS           NO

LVS SPICE STRICT WL                    NO

// LVS SPICE OPTION

LVS STRICT SUBTYPES                    NO

LVS EXACT SUBTYPES                     NO

LAYOUT CASE                            NO

SOURCE CASE                            NO

LVS COMPARE CASE                       NO

LVS DOWNCASE DEVICE                    NO

LVS REPORT MAXIMUM                     50

LVS PROPERTY RESOLUTION MAXIMUM        32

// LVS SIGNATURE MAXIMUM

// LVS FILTER UNUSED OPTION

// LVS REPORT OPTION

LVS REPORT UNITS                       YES

// LVS NON USER NAME PORT

// LVS NON USER NAME NET

// LVS NON USER NAME INSTANCE

// LVS IGNORE DEVICE PIN

// LVS PREFER NETS FILTER SOURCE

// LVS PREFER NETS FILTER LAYOUT

// Reduction

LVS REDUCE SERIES MOS                  YES

LVS REDUCE PARALLEL MOS                YES

LVS REDUCE SEMI SERIES MOS             YES

LVS REDUCE SPLIT GATES                 YES

LVS REDUCE PARALLEL BIPOLAR            YES

LVS REDUCE SERIES CAPACITORS           YES

LVS REDUCE PARALLEL CAPACITORS         YES

LVS REDUCE SERIES RESISTORS            YES

LVS REDUCE PARALLEL RESISTORS          YES

LVS REDUCE PARALLEL DIODES             YES

LVS REDUCTION PRIORITY                 PARALLEL

LVS SHORT EQUIVALENT NODES             NO

CELL COMPARISON RESULTS ( TOP LEVEL )

#       ###################       _   _

#        #                 #       *   *

#   #         #     CORRECT     #         |

# #          #                 #       \___/

#           ###################

LAYOUT CELL NAME:         NAND3

SOURCE CELL NAME:         NAND3

--------------------------------------------------------------------------------------------------------------

INITIAL NUMBERS OF OBJECTS

--------------------------

Layout    Source         Component Type

------    ------         --------------

Ports:              6         6

Nets:              10        10

Instances:          3         3         MN (4 pins)

3         3         MP (4 pins)

------    ------

Total Inst:         6         6

NUMBERS OF OBJECTS AFTER TRANSFORMATION

---------------------------------------

Layout    Source         Component Type

------    ------         --------------

Ports:              6         6

Nets:               8         8

Instances:          1         1         NAND3 (4 pins): output input input input

------    ------

Total Inst:         1         1

**************************************************************************************************************

INFORMATION AND WARNINGS

**************************************************************************************************************

Matched    Matched    Unmatched    Unmatched    Component

Layout     Source       Layout       Source    Type

-------    -------    ---------    ---------    ---------

Ports:               6          6            0            0

Nets:                8          8            0            0

Instances:           1          1            0            0    NAND3

-------    -------    ---------    ---------

Total Inst:          1          1            0            0

o Initial Correspondence Points:

Ports:        VDD VSS A B C OUT

**************************************************************************************************************

SUMMARY

**************************************************************************************************************

Total CPU Time:      16 sec

Total Elapsed Time:  20 sec